POWER SYSTEM SIMULATION METHODOLOGY

Tools: Python (PyPSA) and DIgSILENT PowerFactory

Simulation Scenarios

All studies are performed under the following two scenarios:

Base Case: Existing power system without the integration of new generation.

New Generation Case: Power system with the proposed new generation connected.

All results are compared between these two cases.

# 1. STEADY-STATE SIMULATIONS

## 1.1 Dispatch and Load Flow Analysis

Objective:

Evaluate the impact of new generation on system operation and power flows.

Methodology:

Define generation dispatch in:

PyPSA (optimization and load flow)

DIgSILENT PowerFactory (AC load flow)

Run load flow simulations for:

Base Case

New Generation Case

Outputs:

Bus voltage magnitudes and angles

Active and reactive power flows

Generator power outputs

## 1.2 Voltage Profile Analysis

Objective:

Assess voltage level variations caused by new generation integration.

Methodology:

Extract bus voltage magnitudes for both scenarios.

Compare voltage levels using a radar (spider) plot.

Outputs:

Radar plot of bus voltages

Voltage deviation comparison between cases

## 1.3 N-1 Contingency Analysis

Objective:

Analyze system security under single contingencies.

Methodology:

Perform N-1 contingency analysis using the DIgSILENT Contingency Analysis module for both scenarios.

Evaluate line overloads and voltage violations.

Outputs:

Contingency result matrices

Violation and severity plots

Comparative analysis between scenarios

## 1.4 Short-Circuit Analysis

Objective:

Evaluate the influence of new generation on fault levels.

Methodology:

Calculate short-circuit currents and short-circuit power:

Base Case

New Generation Case

Analyze changes in fault levels at critical buses.

Outputs:

Short-circuit current and power values

Comparative plots and tables

# 2. Small-Signal Stability Analysis

## 2.1 Eigenvalue-Based Small-Signal Stability

Objective:

Determine the impact of new generation on system oscillatory modes.

Methodology:

Perform eigenvalue analysis for:

Base Case

New Generation Case

Identify critical modes and damping ratios.

Outputs:

Eigenvalue plots

Damping ratio comparison

## 2.2 Small-Signal Perturbation Response

Objective:

Assess dynamic response to small disturbances.

Methodology:

Apply a small-signal perturbation (e.g., step change in load or reference power).

Plot generator speed deviations for both scenarios.

Outputs:

Generator speed vs. time plots

Scenario comparison

# 3. Transient Stability Analysis

Objective:

Evaluate system stability under large disturbances.

Methodology:

Apply a three-phase fault at a selected substation.

Clear the fault after a defined clearing time.

Perform simulations for both scenarios.

Outputs:

Bus voltage vs. time

Generator speed vs. time

Generator rotor angle vs. time

Stability comparison between cases

# 4. Voltage Stability Analysis

Objective:

Analyze voltage recovery and reactive power support during faults.

Methodology:

Apply a single-phase fault followed by successful auto-reclosing.

Simulate both scenarios.

Monitor reactive power responses.

Outputs:

Reactive power vs. time

Voltage recovery curves

Comparative analysis

# 5. Frequency Stability Analysis

Objective:

Assess frequency behavior following generation loss.

Methodology:

Simulate disconnection of a generating unit.

Perform simulations for:

Base Case

New Generation Case

Analyze frequency dynamics.

Key Metrics:

System frequency

Rate of Change of Frequency (RoCoF)

Frequency nadir

Generator speed response

Outputs:

Frequency and RoCoF plots

Generator speed vs. time

Comparative performance indicators

# 6. Results Export and Documentation

Requirement:

Export all plots and numerical results for:

Base Case (without new generation)

New Generation Case (with new generation)

Formats:

Plots: PNG / PDF

Numerical results: CSV / Excel

Automated export using Python scripts when applicable